# Driveway Permits on Town Roads

Standard Operating Procedure

- Purpose

To provide a standard procedure for issuing driveway access permits to town roads.  This process will outline the responsibilities of the applicant and town employee involved.  Attached is a copy of the Ordinance to be followed.

- Outline of Process

1.	Person indicates to business office that they would like access to land from town road.  The applicant is given a copy of 1) Standard Operating Procedure and 2) a copy of the Town Driveway Permit and Specifications Ordinance.

2.	Payment of $2100.00 is made to the Town Business office. (See page 3 of the Town Driveway Permits and Specifications Ordinance.) Payment must be made prior to receiving a building permit.

- Person is asked to stake out the area where they would like the access and call (608-825-8420) when ready for site review.  Office notifies highway personnel applicant is ready. Provide map and other information as needed.

4.	Inspection of the site is made by one of the highway patrolmen to determine if the site is appropriate for an access and to determine the size of culvert.

5.	The highway personnel notify the applicant of the culvert and site requirements.

6.	BEFORE Driveway is concreted or paved, applicant must have an inspection to ensure erosion control and 8 inches of fill covers culvert. Applicant must call 608-825-8420 to request inspection.

7.	The applicant is to call 608-825-8420 when the installation of the culvert 	and driveway is complete. One of the highway personnel will make an inspection of the site.

8.	The Driveway Permit form will be signed at that time, indicating that the culvert and driveway are correctly installed.  Copy is given to applicant.

9.	Applicant is reminded that all CONSTRUCTION EQUIPMENT IS TO ACCESS LOT FROM THE DRIVEWAY. The intent is to hold owner responsible for potential damage by construction equipment.

10.	If at the time of the issuance of the occupancy permit by the building inspector, (and after the highway patrolman has approved it), there is no damage to the Town Road, and all erosion control has been maintained throughout the project (ditches and culvert) $2000.00 of the escrowed money will be returned to applicant.

11.	 If in the estimation of the highway patrolman there is damage to the road, or if proper erosion control has not been maintained, the $2000 escrow will be retained by the town until all road repairs are made and ditches and culverts are properly restored. In all instances, $100.00 will be retained by the Town for administrative fees for permit issuance and the inspection process. If the cost of repairing damage exceeds $2000.00, the permit holder will pay the amount exceeding $2000.00.

12.	Zoning permits and Building Permits cannot be issued until the culvert is installed.

## Title 4, Public Works and Safety

### Chapter 4

#### Driveway Permits and Specifications

#### Section 4-4-1:  Permit Required

- No person shall establish or construct a driveway, highway, or

other access, from any parcel to any highway required to be

maintained by the town, without first applying for, and receiving a

driveway permit from the Town Board or its designee (highway

patrolman).  An approved driveway entrance shall be in place

before a building permit shall be issued.

(2)     The highway patrolman shall approve or reject the location and

specifications of the driveway following a review of the application

and the on-site conditions, based on the Dane County driveway

siting criteria in effect on the date of application, the provisions of

this ordinance, and his or her judgment.

(a)      The applicant may appeal the decision of the highway

patrolman to the town board, at a regularly scheduled board

meeting.  The board shall make a final decision only after

having viewed the on-site conditions, and hearing any testimony presented.

(3)    At the time of making an applications for a driveway permit, the applicant shall

pay a permit fee of $100.00 and make an escrow deposit with the Town in the

amount of $2, 000.00 for new construction and $1,000.00 for alterations.  The

intent of this escrow deposit is to hold the owner responsible for potential damage

to the Town Road or right of way or for failure to establish effective erosion

control measures such as planting and growth of grass.  Upon completion of the

drive way construction/alteration the highway patrolman shall inspect the site.  If

there is no damage to the road and effective control measures are established, the

escrow deposit shall be returned to the applicant.  If, in the estimation of the

highway patrolman, there is damage to the road, the road will be repaired and the

cost will be deducted from the escrow deposit.  If the cost of the repair of the

damage exceeds the escrow deposit, the applicant shall pay the difference to the

Town.

(a)      Field access permit holders do not need to escrow $2000.00.

A $100.00 fee shall be paid at the time of the application

#### Section 4-4-2:  Specifications

- Each residential and business lot within the Town of Burke shall be

allowed one driveway entrance.  When appropriate, the Town Board may

stipulate that adjacent lots be served by joint shared driveways.

- Each driveway, regardless of length, which serves a developed or

undeveloped parcel within the town limits, shall have a metal culvert

along the ditch-line, unless an alternate installation is approved by the

highway patrolman.

- The culvert shall be sized such that it will adequately carry the

expected storm water runoff; and shall be the equivalent of a

circular pipe 15 inches or more in diameter.  The Town may

require water flow calculations. Note:  Due to the nature of the

existing highway ditch-lines within the town, consideration

should be given to installing an elliptical culvert pipe to allow

the driveway to meet the pavement with out excessive rise in

elevation, or the need for unnecessary grading along the ditch-

line.

- Each culvert shall have a flared end-section, or a head wall of

earthen material, not concrete or asphalt, at both ends.  The

head walls required by this section, shall be installed so that they

do not create a traffic hazard, or interfere with the snow

removal  operations of the town.  The ends of the culvert

should be sloped at a minimum of 2:1.

(c)  Each culvert shall be 24 feet or more in length, unless the site

conditions justify the highway patrolman to authorize an

alternate length.

- Where driveways of concrete are desired, the applicant must

sign a waiver and accept financial responsibility for repairs to

driveway in the event the town must perform maintenance

within the town right of way.  Driveways must be designed in

such a manner that asphalt is laid for the apron at least 2 feet

from the edge of the road into the driveway.  The intent is to be

able to reconstruct town roads without elevation changes to

the roadbed.  (See design specifications at end of chapter.)

- The driveway within the area of the public road right of way shall

slope away from the public road at a minimum of 1% and a

maximum of 5% to prevent erosion onto the public road.  (See

diagram below.)

- Each driveway which serves a dwelling, commercial building,

agricultural building, garage, or other outbuilding, which is less than 200 feet in length shall be constructed and maintained with:

(a)  A  stabilized base of compacted rock, 6 inches or more in depth;

(b) An all-weather roadway of compacted road gravel, a minimum

of 2 inches of asphalt or 3 inches of concrete, and 10 feet or more

in width;

(c) An area free of encroachments that extends 6 feet or more from

each side of the centerline of the driveway and 12 feet or more in

height;

(d) Adequate drainage, including side ditches as necessary

- An average grade of 10 percent or less.

- Each driveway which serves a dwelling, commercial building,

agricultural building, garage or other outbuilding, which is 200 feet

or more in length shall be constructed and maintained in

accordance with the design standards, as follows:

- A stabilized base of compacted rock, 8 inches or more in depth;

- An all-weather roadway of compacted road gravel a minimum of

2 inches or asphalt or 3 inches of concrete and 12 feet or more in width;

- An area, free of encroachments which extends 6 feet or more from

each side of the centerline of the driveway and 12 feet or more in height;

- Adequate drainage, including side ditches as necessary;

- An average grade of 10 percent or less-Maximum width clearance of 24 feet;

- Maximum commercial drive width is 35 feet;

- Base requirements: crushed 3 in rock………….5 inches

¾ inch crushed rock………3 inches

Total                                    8 inches

(6)  All cost of construction, including but not limited to: the cost of the

culvert, flared end-sections or head-walls, and any grading and paving required,

shall be the responsibility of the landowner of the parcel the driveway serves.

(7) All construction equipment shall use the prepared driveway to access the lot.  Road

damage caused by equipment shall be the responsibility of the lot owner.

(8)  Replacements of existing culverts shall require a 24-foot metal culvert

with flared end-sections or head walls as described in Section 4-4-2 of

this ordinance.

#### Section 4-4-3: Definitions

(1) “Culvert” means any device for the drainage of water that is installed

under or along, a highway or driveway within the town limits.  A culvert

may be constructed of metal or concrete.

(2) “Driveway or private road” has the meaning specified in S. 340.01(46), Stats.

(3)  “Highway” has the meaning specified in S. 340.01(22), Stats.

(4)  “Highway Patrolman” means the person designated by the Town Board with

the responsibilities specified in S. 81.03, Stats.

(5)  “Right of Way” has the meaning specified in S. 340.01(51), Stats.

(6)  “Roadway” has the meaning specified in S. 340.01(54), Stats. and may be

applied to driveways and highways alike.

##### Section 4-4-4:   Enforcement

(1)  Any person who violates any provision of this section shall be subject to

a forfeiture of not less than $100.00 or more than $500.00 for each

violation.

(2)  Any driveway constructed without a permit is declared to be a public

nuisance, and may be abated by the Town.  The cost of such abatement may be

charged  to the property owner under WI Statutes Sec. 66.60.